LAB 1

MUX

(2-to-1)

a1-a4

output(a1-a4/b1-b4)

b1-b4

sel

Figure. 1

Objective: Code the behavior of a 2-to-1 MUX in VHDL.

Logic: A multiplexer or a data selector is a combinational circuit that selects binary information from one of many input lines and directs it to a single output line. In figure 1, we see two input lines and one output line each of four bits wide.

The selection of a particular input line is controlled by a set of control/selection lines (‘sel’). Generally, there are 2n input lines and n selection lines.

| Operation: sel | output |
| --- | --- |
| ‘0’ | a1-a4 |
| ‘1’ | b1-b4 |

Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity mux is

Port (

--First set of inputs

a1,a2,a3,a4   	:in std_logic;--a1 is MSB

--Second set of inputs

b1,b2,b3,b4	 :in std_logic;--b1 is MSB

--Selection line

sel		 :in std_logic;

--Output from the MUX

output		:out	std_logic_vector(1 to 4)

);

end entity mux;

architecture Behavioral of mux is

begin

Multiplex: process(sel)

begin

if(sel='0') then

output(4)<=a4; output(3)<=a3; output(2)<=a2; output(1)<=a1;

else

end if;

end process Multiplex;

end Behavioral;

LAB 2

(MSB)				(LSB)

a3                            b3	                      a2                            b2	 a1                            b1

carry_prop3

carry_gen3

carry_prop2

carry_gen2

carry_prop1

carry_gen1

‘00'	‘0’

carry3                                                 carry2                                                carry1

output(4)                                           output(3)                                               output(2)                                         output(1)

Figure. 2

Objective: Code the behavior of 3 bit carry look-ahead adder in VHDL.

Logic: Refer to Course Reader.

The terms Gi, Pi, Ci, and Si used in the reader has been represented here as carry_geni, carry_propi, carryi, and output(i) respectively.

Operation:  The carry look-ahead adder depicted in figure 2 is in relation with the behavioral description given below

Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity adder_lookahead is

Port(

a1,a2,a3	:in std_logic;--a3 is  MSB

b1,b2,b3	:in std_logic;--b3 is MSB

output		:out std_logic_vector(4 downto 1)  --Sum

);

end entity;

architecture Behavioral of adder_lookahead is

signal carry_prop1,carry_prop2,carry_prop3                	  :std_logic;

signal carry_gen1,carry_gen2,carry_gen3                    	 :std_logic;

signal carry1,carry2,carry3                              		 :std_logic;

begin

process(a3,a2,a1,b3,b2,b1)

begin

carry_prop1<=a1 xor b1; carry_gen1<=a1 and b1;

carry_prop2<=a2 xor b2; carry_gen2<=a2 and b2;

carry_prop3<=a3 xor b3; carry_gen3<=a3 and b3;

end process;

process(carry_prop1,carry_gen1)

begin

carry1<=carry_gen1 or (carry_prop1 and '0');

output(1)<=carry_prop1 xor '0';

end process;

process(carry1)

begin

carry2<=carry_gen2 or (carry_prop2 and carry1);

output(2)<=carry_prop2 xor carry1;

end process;

process(carry2)

begin

end process;

process(                         )

begin

--hint: carry from the previous stage would form the MSB of the sum [output(4)]

end process;

end Behavioral;

LAB 3

‘0’                                         ’1’                                                                                                                                                            ‘0’,’1’

1

2

3

4

5

‘1’	‘0’	   ‘0’	‘1’

‘0’

‘1’

Figure. 3

Objective: Code the behavior of the state machine described above in VHDL. This type of state machine is called

Logic: The state machine described above has five states. The final state (S5) is reached when the pattern “1001” is observed in the input stream.

Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity state_machine is

Port (

input  			:in std_logic;

output_state		:out std_logic_vector(2 downto 0);

cclk			:in std_logic; --CLOCK

reset			:in std_logic;

accept_input		:in std_logic

);

end entity state_machine;

architecture Behavioral of state_machine is

type STATE_TYPE is (S1,S2,S3,S4,S5);

signal STATE,NEXTSTATE : STATE_TYPE ;

begin

machine: process (input,accept_input,STATE) begin

NEXTSTATE <= STATE ;

case STATE is

when S1     =>

if accept_input='1' then

if input='0' then

NEXTSTATE<=S1;

else

NEXTSTATE<=S2;

end if;

end if;

when S2     =>

if accept_input='1' then

if input='0' then

NEXTSTATE<=S3;

else

NEXTSTATE<=S2;

end if;

end if;

when S3	     =>

if accept_input='1' then

if input='0' then

NEXTSTATE<=S4;

else

NEXTSTATE<=S2;

end if;

end if;

when S4	      =>

when S5	      =>

end case ;

end process machine ;

output_state <=  "001" when STATE=S1 else

"010" when STATE=S2  else

"011" when STATE=S3  else

"100" when STATE=S4 else

"101" when STATE=S5;

Reg: process (cclk,reset)

begin

if reset='1' then

STATE <= S1 ;

elsif cclk'event and cclk='1' then

STATE <= NEXTSTATE ;

end if ;

end process Reg;

end Behavioral ;

LAB 4

clk

COUNTER

(4-bit binary up/down)

count_value	4				      4	output

select_up_down               reset

Figure. 4

Objective: Code the behavior of a 4-bit binary up/down counter in VHDL.

Logic: A counter is a combinational circuit that tallies or counts the number of input pulses (clk). Counters are fundamental components of digital systems. Digital counters are used in wide variety of applications, including pulse counting, frequency division, time measurement, control, and other timing operations.

Figure 4, depicts an up/down configurable 4-bit binary counter. The maximum count or the minimum count value can be set for up count or down count respectively

Operation:

| select_up_down | operation | count_value | count -sequence |
| --- | --- | --- | --- |
| ‘1’ | Count-up | X (max count) | counts up from “0000” to X |
| ‘0’ | Count-down | X(min count) | counts down from “1111” to X |

X- Any 4-bit binary combination

Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity counter is

Port (

count_value			:in std_logic_vector(3 downto 0);--Max or Min value

select_up_down			:in std_logic; --'1' is up counter

output				:out std_logic_vector(3 downto 0);

clk				:in std_logic;--CLOCK

reset				:in std_logic

);

end entity counter;

architecture Behavioral of counter is

signal count :std_logic_vector(3 downto 0):= ”0000”;--Initialization

begin

process(clk)

begin

if(clk='1'and clk'event) then

if (reset='1') then

count<=(others=>'0');

elsif(select_up_down='1') then --count UP

if(count=count_value) then

count<=(others=>'0');

else

count<=count+1;

end if;

else--count DOWN

if(count=count_value) then

count<=(others=>'1');

else

count<=count-1;

end if;

end if;

end if;

end process;

Question) This signal assignment statement can be placed only after the process?

Answer)

- Yes. As count is being modified inside the process, this signal assignment statement has to be after the execution of the process.

- No, location of any signal assignment statement (that are outside the process) does not matter as VHDL is event driven.

output<=count;

end Behavioral;

LAB 5

clk       reset

input1-input4           4                                                            4	         output(1)-output(4)

4-bit Shift

Register

reg(1)-reg(4)

input_accept

‘1’

siso or sipo or piso or piso

Figure. 5

Objective: Code the behavior of a 4-bit shift register in VHDL.

Logic: Shift registers are used in digital systems for temporary information storage and for data manipulation or transfer.  There are two ways to shift (store) data into a register i.e. serial or parallel, and similarly two ways to shift data out of the register. The following are the four basic modes of operation:

MSB

Shift register   (4 bits)

Shift register   (4 bits)

serial data input                                serial data output                         serial data input	                   parallel

data outputs

LSB

a. serial in-serial out (siso)                       	                                         b. serial in-parallel out (sipo)

Shift register   (4 bits)

Shift register   (4 bits)

MSB	  MSB                              MSB

parallel                                                     serial data output                     parallel	      parallel

data inputs                                                                                                data inputs                                                               data outputs

LSB	 LSB	 LSB

c. parallel in-serial out (piso)  				             d. parallel in-parallel out (pipo)

a. Serial in-Serial out: In this type of shift register, data is stored into the register one bit at a time (serial) and taken out serially too.

b. Serial in-Parallel out: Here data is stored serially into the register and is taken out collectively at one shot (parallel).

c. Parallel in-Serial out: In this case entire data is stored into the register in one shot and is taken out serially.

d. Parallel in- Parallel out: Data is stored into the register at one go and is taken out collectively at one shot too.

| Selection | Operation |
| --- | --- |
| If siso=’1’ | Serial in-Serial out |
| if sipo=’1’ | Serial in-Parallel out |
| If piso=’1’ | Parallel in-Serial out |
| If pipo=’1’ | Parallel in-Parallel out |

Operation:

Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity shift_register is

Port (

input1,input2,input3,input4  :in std_logic;--input 1 is MSB

reg					     :inout std_logic_vector(1 to 4);

output                                   :inout std_logic_vector(1 to 4);

input_accept	                     :in std_logic;

siso			             :in std_logic;

sipo			             :in std_logic;

piso			             :in std_logic;

pipo			             :in std_logic;

reset			             :in std_logic;

clk			                   : in std_logic

);

end shift_register;

architecture Behavioral of shift_register is

signal count:std_logic_vector(2 downto 0):="000";

begin

SHIFT_REG: process(clk)

begin

if clk'event and clk='1' then

--Serial in-Serial out

if siso='1' then

if(reset='1') then

count<="000";

reg<="0000";

output<="0000";

end if;--reset

if input_accept='1' then

--Start serial in

if count="000" then

count<=count+1;

reg<=input4 & "000";--(others=>'0');

elsif count="001" then

count<=count+1;

reg<=input3 & reg(1 to 3);

elsif count="010" then

count<=count+1;

reg<=input2 & reg(1 to 3);

elsif count="011" then

count<=count+1;

reg<=input1 & reg(1 to 3);

----Start serial out

elsif count="100" then

count<=count+1;

output<=reg(4) & "000";--(others=>'0');

reg<='0' & reg(1 to 3);

elsif count="101" then

count<=count+1;

output<=reg(4) & output(1 to 3);

reg<='0' & reg(1 to 3);

elsif count="110" then

output<=reg(4) & output(1 to 3);

reg<='0' & reg(1 to 3);

else

output<=reg(4) & output(1 to 3);

reg<='0' & reg(1 to 3);

end if;

end if;--input_accept

---Serial in-Parallel out

elsif sipo='1' then

if(reset='1') then

count<="000";

reg<="0000";

output<="0000";

end if;--reset

if input_accept='1' then

--Start serial in

if count="000" then

count<=count+1;

reg<=input4 & "000";--(others=>'0');

elsif count="001" then

count<=count+1;

reg<=input3 & reg(1 to 3);

elsif count="010" then

count<=count+1;

reg<=input2 & reg(1 to 3);

elsif count="011" then

count<=count+1;

reg<=input1 & reg(1 to 3);

--Start parallel out

else

output<=reg;

reg<="0000";

end if;

end if;--input_accept

--Parallel in-Serial out

elsif piso='1' then

if(reset='1') then

count<="000";

output<="0000";

reg<="0000";

end if;--reset

if input_accept='1' then

--Start  parallel in

if count="000" then

count<=count+1;

reg<=input1 & input2 & input3 & input4;

---Start serial out

elsif count="001" then

count<=count+1;

output<=reg(4) & "000";--(others=>'0');

reg<='0' & reg(1 to 3);

elsif count="010" then

count<=count+1;

output<=reg(4) & output(1 to 3);

reg<='0' & reg(1 to 3);

elsif count="011" then

count<=count+1;

output<=reg(4) & output(1 to 3);

reg<='0' & reg(1 to 3);

else

output<=reg(4) & output(1 to 3);

reg<='0' & reg(1 to 3);

end if;

end if;--input_accept

--Parallel in-Parallel out

elsif pipo='1' then

if(reset='1') then

count<="000";

output<="0000";

reg<="0000";

end if;

if input_accept='1' then

--Start parallel in

if count="000" then

count<=count+1;

reg<=input1 & input2 & input3 & input4;

else

--Start parallel out

output<=reg;

reg<="0000";

end if;

end if;--input_accept;

end if;--end of select1,select2,select3,select4

end if;-- clk

end process SHIFT_REG;

end Behavioral;