• VHDL uses signals to define a data pathway between two functional units

Although VHDL offers both variables and signals, we will use mainly signals

-Variables:

Data objects that can be assigned a current value

Assignment statements execute sequentially, 'in-line' with the code

Variable values are not scheduled

Target values update immediately upon execution of assignment stmt.

Do not correlate well with H/W constructs which execute concurrently

-Signals:

Data objects that can be assigned a time series of values

i.e., can specify (value, time) pairs for the data object

Assignment statements simulate concurrent execution

Not necessarily 'In-Line' execution order

Signal values are scheduled

Target values are updated only after a wait time lapses (if specified)

Correlates to the data on physical hardware wires

Serves as a communication path from one component to another

Signals connect components at their ports.

• Signal Assignment Statement

General form is:

signal-name   <= 	value after time ;

where: 	value is a bit type or a computed logical Boolean expression

Bit Type is '0' or '1' (note: single quotes are required)

Logical operators in VHDL include: and, or, nand, nor, xor, not

time is an integer number of time units in VHDL (we will use ns)

Used to specify the delay before updating the target signal

Models propagation time of logic gates or other digital circuitry

(note: units are required; also, ns needs a leading blank space)

Examples:

X <= '0' ;

Y <= '1' ;

Sum <= '1' after 5 ns ;

Carry <= '0' after 8 ns ;

If no after time specified, it defaults to after 0 ns

i.e., it executes and updates immediately at the current simulation time

If an after time is specified, it is relative to the current simulation time

So, if executed at time = 7 ns and delay is 5 ns, signal changes at 12 ns

• Current Simulation Time

The virtual time 'pointer' of the simulator clock

System state at current simulation time gives a snapshot of it in the simulation

• Simulation Time:

Based on the concept of discrete events and event-driven simulation

Discrete-event simulation:

State variables change only at a countable number of points in time

These points in time are the ones at which an event occurs

Event: An instantaneous signal change which can alter the state of the system

Simulation time does not advance in normal 'analog' or 'digital' fashion

Simulator clock does not use constant, fixed increments

Skips periods of no activity; Jumps to next most imminent event time

A statement can 'go' if any of its RHS signals change value at the current sim time

Its target value can only change if any of the signals it depends upon changes

A statement with no right-hand-side (RHS) signals will 'go' at simulation time 0

It does not depend on any other signals; 'Goes' once at simulation startup

Time specification in this case can be construed as absolute simulation time

When a signal assignment statement 'goes', it posts an event for the target signal

• Event Queue

Used by the VHDL simulator to keep track of scheduled signal changes (events)

Stores a sorted list of events in time ascending order

There can be multiple events scheduled for the same time

So, the above 4 signal assignment statements produce the following events

| Time | 0 | 5 | 8 |
| --- | --- | --- | --- |
|  | X: ('0', 0) | Sum: ('1', 5) | Carry: ('0', 8) |
|  | Y: ('1', 0) |  |  |

• A target signal's value can depend on other signals specified on the RHS

A signal assignment statement is activated (can 'go') upon a RHS event

i.e., whenever a RHS signal changes in value

Example:			Q <= '1' after 2 ns ;

QBAR <= not Q after 5 ns ;

| Time | 2 | 7 |
| --- | --- | --- |
|  | Q: ('1', 2) | QBAR: ('0', 7) |

• Multiple (value, time) pairs can be assigned in a single statement

Enables any arbitrarily shaped waveform to be generated

General form is:

signal-name   <= 	value after time ,

value after time ,

...

value after time ;

Example:

Q  <=	'1' after 0 ns,

'0' after 15 ns,

'1' after 33 ns,

'0' after 38 ns,

'1' after 48 ns;

Since this assignment statement does not depend on any signals on RHS,

it is activated only once at startup (i.e., at simulation time = 0)

Entire series of events is posted to event queue when simulation run begins

Last value specified is steady state value ('1' for all time after 48 ns)

Creates the waveform below.

• An assignment statement dependent on a RHS signal goes whenever RHS has event

Example:

QBAR <= not Q after 2 ns ;

Creates a negated Q which lags Q by 2 ns.

• Note: It is important to initialize signal values at simulator startup

All signals are initialized to '0' at simulation time 0

Delay time of dependent signal is not 'rolled back into negative time' by simulator

Need to initialize value of QBAR via a signal declaration statement

• Note: Above event queue is 'simplified' inertial delay model

• Delays

Controls effect of a signal assignment which is dependent on RHS signal changes

Two types in VHDL:

- Inertial Delay

The default mode for VHDL simulator

Models components using a minimum "setup and hold" time

Value on inputs must persist for given time before output responds

Useful in limiting 'spikes' associated with certain circuits

e.g.) Limits transients of a flip-flop in the process of switching

- Transport Delay

Specified using a TRANSPORT keyword in the signal assignment stmt

e.g.)  	signal-name   <= transport value after time ;

Similar to wire or transmission time delay

Output always changes regardless of time duration of input signal

Useful when more detailed examination of each simulation time step reqd

Good for observing transient response of output

High speed inputs without minimum steady state can be modeled

Can provide the equivalent of macroscopic circuit simulation

• For Inertial Delay, important to ensure that RHS signals persist for at least delay time

e.g.)	QBAR <= not Q after 10 ns ;

• For transport delay, important to realize that target signal's value computed at CST

The computed value is just not assigned to the target signal until after the delay

Contrast with: Wait for specified delay, then read RHS signals and update output